Melaku  Desalegn

ECE510- Spring 2025

Week2

Design, test, and benchmark a co-processor chaplet that accelerates parts of some AL. ML code /algorithm for Time series analysis for weather forecasts.

### 1. What are you trying to do?

We want to build a small, specialized computer chip that can make weather forecasts faster and more efficiently. It works alongside a regular computer to speed up the parts of the weather prediction process that take the most time.

### 2. How is it done today, and what are the limits of current practice?

Right now, weather forecasting with machine learning is usually done on regular CPUs or GPUs. These systems are powerful, but they’re not optimized for the repetitive and time-based nature of weather data. This can lead to high power use, longer wait times, and less efficient scaling—especially when running models on large amounts of local weather data (like for cities or farms).

### 3. What is new in your approach and why do you think it will be successful?

We’re creating a custom chiplet that is purpose-built for the kind of math used in time-based weather prediction models like GRUs or Transformers. Unlike general-purpose chips, this one only focuses on what’s needed—like handling time sequences and doing small, fast calculations. It’s smaller, uses less power, and can sit close to the main computer or even inside edge devices (like weather stations). That focus gives us speed, energy savings, and cost benefits.

### 4. Who cares? What difference will it make if you succeed?

- Weather services could run forecasts more often and at lower cost.

- Farmers and renewable energy operators could get more accurate local predictions in real-time.

- Developing regions could run advanced models on low-power hardware.

- Research labs could run massive simulations faster, with lower cloud compute bills.

### 5. What are the risks?

- The chiplet might not outperform optimized GPUs if not carefully designed.

- Adapting existing ML models to run on the chiplet could be complex.

- There’s a risk that energy savings don’t offset development cost unless used at scale.

- Fabrication delays or bugs could slow progress.

### 6. How much will it cost?

- Prototype development (FPGA or emulation): ~$50K

- First silicon test chiplet (with shuttle runs): ~$100K–$250K

- Full product development (design + software stack): ~$500K–$1M

- Optional: Additional $100K+ for integration with cloud platforms or edge hardware

### 7. How long will it take?

- Design & simulation (python/HLS): 3–4 months

- FPGA prototype testing: 2–3 months

- Chiplet tape out & fabrication: 3–5 months

- Software stack & runtime (ONNX adapter, etc.): parallel over 6–8 months

- Total time to first deployment-ready chiplet: ~12 months

### 8. What are the mid-term and final “exams” to check for success?

#### Mid-Term Milestones:

- FPGA-based chiplet emulation outperforms CPU baseline for GRU inference

- Weather forecast model converted to ONNX runs on chiplet with >90% accuracy retained

- Power usage <50% of equivalent GPU workload

#### Final Exams:

- End-to-end weather model (e.g., 30-day sequence) runs 3× faster and 5× more energy-efficient than on CPU

- Integration tested with real datasets (e.g., NOAA, WeatherBench)

- Deployed in at least one real-world environment (research station, drone, or farm)

1. Functional Focus:

- Accelerate GRU, LSTM, and Transformer-style attention operations, especially:

  - Matrix-vector multiplies for sequential data

  - Softmax + layer norm optimizations

  - Quantized activations to reduce memory and bandwidth pressure

2. Chiplet Architecture:

- Memory-near-compute: Local SRAM for time-series window buffers

- Lightweight scheduler for sequence ops

- On-chip accelerator cores: 4–8 small vector engines with support for time-step parallelism

- IO support: PCIe/CXL or tile-based interconnect

3. Software Stack Outline:

- Runtime for scheduling sequences

- ONNX runtime compatibility layer

- Quantization-aware training pipeline (optional but helpful)

Draw for me an algorithm for weather forecast.

A code for simulating the diagram in Verilog.

create a basic Verilog simulation that mimics the data flow shown in the diagram:

- Weather Data comes into a regular CPU (modeled as a testbench).

- It sends time series data to a Weather Forecast Accelerator.

- The accelerator processes it (simple logic) and sends back weather predictions.

### Simulation Scope

For simplicity:

- We'll simulate 1D time series data as an input.

- The "forecast accelerator" will be a placeholder that performs a moving average (simulating simple model behavior).

## Verilog Modules

### 1. forecast_accelerator.v — Simple Moving Average Module

verilog

CopyEdit

module forecast_accelerator(

input clk,

input rst,

input [7:0] data_in,

input       valid_in,

output reg [7:0] data_out,

output reg       valid_out

);

reg [7:0] buffer[2:0];  // 3-sample window

integer i;

always @(posedge clk or posedge rst) begin

if (rst) begin

for (i = 0; i < 3; i = i + 1)

buffer[i] <= 0;

data_out <= 0;

valid_out <= 0;

end else if (valid_in) begin

// Shift buffer

buffer[2] <= buffer[1];

buffer[1] <= buffer[0];

buffer[0] <= data_in;

// Simple moving average output

data_out <= (buffer[0] + buffer[1] + buffer[2]) / 3;

valid_out <= 1;

end else begin

valid_out <= 0;

end

end

endmodule

### 2. testbench.v — Simulates "Regular Computer."

verilog

CopyEdit

`timescale 1ns/1ps

module testbench;

reg clk = 0;

reg rst = 1;

reg [7:0] data_in;

reg       valid_in;

wire [7:0] data_out;

wire       valid_out;

// Instantiate Accelerator

forecast_accelerator accel (

.clk(clk),

.rst(rst),

.data_in(data_in),

.valid_in(valid_in),

.data_out(data_out),

.valid_out(valid_out)

);

// Clock generation

always #5 clk = ~clk;

integer i;

reg [7:0] test_data[9:0];

initial begin

// Simulated weather data (e.g., temperature or pressure values)

test_data[0] = 8'd20;

test_data[1] = 8'd22;

test_data[2] = 8'd23;

test_data[3] = 8'd25;

test_data[4] = 8'd24;

test_data[5] = 8'd26;

test_data[6] = 8'd27;

test_data[7] = 8'd29;

test_data[8] = 8'd28;

test_data[9] = 8'd30;

#10 rst = 0;  // Release reset

for (i = 0; i < 10; i = i + 1) begin

@(posedge clk);

data_in = test_data[i];

valid_in = 1;

end

@(posedge clk);

valid_in = 0;

#100 $finish;

end

// Monitor output

always @(posedge clk) begin

if (valid_out)

$display("At time %t: Forecast = %d", $time, data_out);

end

endmodule

Output

Hardware Block Diagram – Weather Forecast Co-Processor Chiplet.